Assignment - 1 Simple CPU Design using Verilog

A Lab Report submitted by

Palak Mishra

B24EE1050

Indian Institute of Technology Jodhpur

Objective :

The goal of this project is to create and validate a compact CPU system using Verilog HDL and Xilinx Vivado. This system features a 16-entry instruction memory, a combinational CPU/ALU that can perform six distinct operations, and an integrated processor that links the instruction memory with the CPU. The entire design is confirmed through behavioral simulation in Vivado.

Memory Specification :

| Parameter | Specification |
| --- | --- |
| Number of locations | 16 |
| Address width | 4 bits |
| Instruction width | 20 bits |
| Read | Combinational |
| Write | Synchronous |
| Write enable | we |

Instruction Format :

20-bit instruction

| Opcode | Operand 1 | Operand 2 |
| --- | --- | --- |
| 4 bits | 8 bits | 8 bits |

Opcode table :

| Opcode | Operation |
| --- | --- |
| 0000 | ADD |
| 0001 | SUB |
| 0010 | AND |
| 0011 | OR |
| 0100 | XOR |
| 0101 | MUL |
| Others | Result = 0 |

Memory initialisation table :

| Address | Instruction |
| --- | --- |
| 0 | ADD 10, 20 |
| 1 | SUB 30, 5 |
| 2 | AND 15, 7 |
| 3 | OR 12, 3 |
| 4 | XOR 10, 5 |
| 5 | MUL 6, 7 |

Source Code : https://github.com/palakmishra01/COA_LAB/tree/main

Vivado simulation result :

Data path : Address -> Instruction Memory -> 20-bit Instruction -> Instruction Extraction -> Opcode + Operand A + Operand B -> CPU / ALU -> 8-bit Result

Console log :

Conclusion :

The integrated processor, the combinational CPU/ALU, and the 16-entry instruction memory were successfully implemented using Verilog HDL in Xilinx Vivado. The ADD, SUB, AND, OR, XOR, and MUL operations were checked by means of the integrated testbench, and all six test cases produced the expected results, which shows that the entire processor datapath functioned correctly.